7.Design and Analysis of FIR Filters Using Parks–McClellan and Frequency-Sampling Techniques Using MATLAB

Objectives of this Experiment:

1. To design FIR filters using the Parks–McClellan equi-ripple method and frequency-sampling technique.

2. To obtain and analyze the FIR coefficients and desired frequency response for each method.

3. To determine passband/stopband ripple and transition characteristics from the simulated responses.

4. To compare the two FIR design methods using common frequency specifications.

PROGRAM:

clear; clc; close all;

Fs = 10000;

fp = 1500;

fs = 2000;

N = 40; % Filter order

% 1. Parks-McClellan (Equiripple) Design

f_bands = [0, fp, fs, Fs/2] / (Fs/2); % Normalized frequency band edges

a_bands = [1, 1, 0, 0];               % Desired amplitudes

b_pm = firpm(N, f_bands, a_bands);

% 2. Frequency-Sampling Design

b_fs = fir2(N, f_bands, a_bands);

% ==========================================

% DISPLAY NUMERICAL OUTPUTS IN COMMAND WINDOW

% ==========================================

fprintf('=== EXPERIMENT 7: PARKS-MCCLELLAN vs FREQUENCY SAMPLING ===\n\n');

fprintf('Filter Order (N): %d\n', N);

fprintf('Passband Edge: %d Hz, Stopband Edge: %d Hz\n\n', fp, fs);

fprintf('--- PARKS-MCCLELLAN COEFFICIENTS (b_pm, First 10) ---\n');

disp(b_pm(1:10));

fprintf('--- FREQUENCY-SAMPLING COEFFICIENTS (b_fs, First 10) ---\n');

disp(b_fs(1:10));

% Frequency Responses

[H_pm, f] = freqz(b_pm, 1, 1024, Fs);

[H_fs, ~] = freqz(b_fs, 1, 1024, Fs);

% Plotting

figure('Name', 'Parks-McClellan vs Frequency-Sampling');

subplot(2,1,1);

plot(f, 20*log10(abs(H_pm)), 'b', 'LineWidth', 1.5); hold on;

plot(f, 20*log10(abs(H_fs)), 'r--', 'LineWidth', 1.5); grid on;

legend('Parks-McClellan (Equiripple)', 'Frequency-Sampling');

title(['FIR Filter Magnitude Response Comparison (Order N = ', num2str(N), ')']);

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)'); ylim([-80 10]);

subplot(2,1,2);

plot(f, angle(H_pm)*180/pi, 'b', 'LineWidth', 1.5); hold on;

plot(f, angle(H_fs)*180/pi, 'r--', 'LineWidth', 1.5); grid on;

legend('Parks-McClellan', 'Frequency-Sampling');

title('Phase Response Comparison');

xlabel('Frequency (Hz)'); ylabel('Phase (degrees)');
OUTPUT:

RESULT:

=== EXPERIMENT 7: PARKS-MCCLELLAN vs FREQUENCY SAMPLING ===

Filter Order (N): 40

Passband Edge: 1500 Hz, Stopband Edge: 2000 Hz

--- PARKS-MCCLELLAN COEFFICIENTS (b_pm, First 10) ---

-0.0015    0.0072    0.0053   -0.0010   -0.0081   -0.0072    0.0037    0.0142    0.0100   -0.0090

--- FREQUENCY-SAMPLING COEFFICIENTS (b_fs, First 10) ---

0.0000    0.0001    0.0002   -0.0001   -0.0008   -0.0009    0.0008    0.0035    0.0029   -0.0039