SIMULATION EXPERIMENT – 2

2. Design and Performance Analysis of Chebyshev Type-I IIR Low-Pass and High-Pass Filters Using MATLAB

Objectives of this Experiment:

1. To design Chebyshev Type-I IIR low-pass and high-pass filters for specified digital frequency requirements.

2. To study the effect of passband ripple on filter order and frequency response.

3. To obtain and analyze filter coefficients, magnitude and phase responses.

4. To compare the Chebyshev Type-I response with the corresponding Butterworth response.

Program Code:

clear; clc; close all;

% Design Parameters

Fs = 10000;         % Sampling frequency in Hz

Ap = 0.5;           % Passband ripple in dB

As = 30;            % Stopband attenuation in dB

% 1. Low-Pass Filter Design

fp_lp = 1500;       % Passband frequency (Hz)

fs_lp = 2000;       % Stopband frequency (Hz)

wp_lp = fp_lp / (Fs/2);

ws_lp = fs_lp / (Fs/2);

[N_lp, Wn_lp] = cheb1ord(wp_lp, ws_lp, Ap, As);

[b_lp, a_lp] = cheby1(N_lp, Ap, Wn_lp, 'low');

% 2. High-Pass Filter Design

fp_hp = 2500;       % Passband frequency (Hz)

fs_hp = 1800;       % Stopband frequency (Hz)

wp_hp = fp_hp / (Fs/2);

ws_hp = fs_hp / (Fs/2);

[N_hp, Wn_hp] = cheb1ord(wp_hp, ws_hp, Ap, As);

[b_hp, a_hp] = cheby1(N_hp, Ap, Wn_hp, 'high');

% ==========================================

% DISPLAY NUMERICAL OUTPUTS IN COMMAND WINDOW

% ==========================================

fprintf('=== EXPERIMENT 2: CHEBYSHEV TYPE-I IIR FILTERS ===\n\n');

fprintf('--- CHEBYSHEV LOW-PASS FILTER ---\n');

fprintf('Calculated Filter Order (N_lp): %d\n', N_lp);

fprintf('Passband Edge Cutoff Frequency (Wn_lp): %.4f Hz\n', Wn_lp * (Fs/2));

fprintf('Numerator Coefficients (b_lp):\n');

disp(b_lp);

fprintf('Denominator Coefficients (a_lp):\n');

disp(a_lp);

fprintf('--- CHEBYSHEV HIGH-PASS FILTER ---\n');

fprintf('Calculated Filter Order (N_hp): %d\n', N_hp);

fprintf('Passband Edge Cutoff Frequency (Wn_hp): %.4f Hz\n', Wn_hp * (Fs/2));

fprintf('Numerator Coefficients (b_hp):\n');

disp(b_hp);

fprintf('Denominator Coefficients (a_hp):\n');

disp(a_hp);

% Frequency Response Calculation

[H_lp, f] = freqz(b_lp, a_lp, 1024, Fs);

[H_hp, ~] = freqz(b_hp, a_hp, 1024, Fs);

% Plotting

figure('Name', 'Chebyshev Type-I LPF and HPF Responses');

subplot(2,2,1);

plot(f, 20*log10(abs(H_lp)), 'b', 'LineWidth', 1.5); grid on;

title(['Chebyshev LPF Magnitude (Order N = ', num2str(N_lp), ')']);

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)'); ylim([-60 5]);

subplot(2,2,2);

plot(f, angle(H_lp)*180/pi, 'r', 'LineWidth', 1.5); grid on;

title('Chebyshev LPF Phase Response');

xlabel('Frequency (Hz)'); ylabel('Phase (degrees)');

subplot(2,2,3);

plot(f, 20*log10(abs(H_hp)), 'b', 'LineWidth', 1.5); grid on;

title(['Chebyshev HPF Magnitude (Order N = ', num2str(N_hp), ')']);

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)'); ylim([-60 5]);

subplot(2,2,4);

plot(f, angle(H_hp)*180/pi, 'r', 'LineWidth', 1.5); grid on;

title('Chebyshev HPF Phase Response');

xlabel('Frequency (Hz)'); ylabel('Phase (degrees)');

OUTPUT:

RESULT:

=== EXPERIMENT 2: CHEBYSHEV TYPE-I IIR FILTERS ===

--- CHEBYSHEV LOW-PASS FILTER ---

Calculated Filter Order (N_lp): 6

Passband Edge Cutoff Frequency (Wn_lp): 1500.0000 Hz

Numerator Coefficients (b_lp):

0.0006    0.0038    0.0095    0.0127    0.0095    0.0038    0.0006

Denominator Coefficients (a_lp):

1.0000   -3.7650    6.8198   -7.3274    4.8717   -1.8927    0.3368

--- CHEBYSHEV HIGH-PASS FILTER ---

Calculated Filter Order (N_hp): 6

Passband Edge Cutoff Frequency (Wn_hp): 2500.0000 Hz

Numerator Coefficients (b_hp):

0.0117   -0.0704    0.1761   -0.2348    0.1761   -0.0704    0.0117

Denominator Coefficients (a_hp):

1.0000    1.3568    2.1347    1.8511    1.2978    0.5937    0.1650