SIMULATION EXPERIMENT – 4

Design and Comparative Analysis of Butterworth, Chebyshev and Elliptic IIR Filters Using MATLAB

Objectives of this Experiment:

1. To design Butterworth, Chebyshev Type-I and Elliptic IIR filters for comparable specifications.

2. To examine passband/stopband ripple, filter order and transition bandwidth for the three approximations.

3. To obtain and compare magnitude responses and filter orders.

4. To evaluate the trade-off between ripple, transition sharpness and filter complexity.

PROGRAM CODE:

clc;

clear;

close all;

%% 1. INPUT PARAMETERS

Fs  = 1000;     % Sampling frequency (Hz)

Fp  = 100;      % Passband edge (Hz)

Fst = 150;      % Stopband edge (Hz)

Rp  = 1;        % Passband ripple (dB)

Rs  = 40;       % Stopband attenuation (dB)

% Normalized frequencies

Wp  = Fp/(Fs/2);

Wst = Fst/(Fs/2);

%% 2. CHEBYSHEV TYPE-I FORMULA

epsilon = sqrt(10^(Rp/10)-1);

N_formula = acosh(sqrt((10^(Rs/10)-1)/(10^(Rp/10)-1))) / acosh(Fst/Fp);

N_theory = ceil(N_formula);

%% 3. BUTTERWORTH FILTER

[Nb,Wb] = buttord(Wp,Wst,Rp,Rs);

[bB,aB] = butter(Nb,Wb,'low');

%% 4. CHEBYSHEV TYPE-I FILTER

[Nc,Wc] = cheb1ord(Wp,Wst,Rp,Rs);

[bC,aC] = cheby1(Nc,Rp,Wc,'low');

%% 5. ELLIPTIC FILTER

[Ne,We] = ellipord(Wp,Wst,Rp,Rs);

[bE,aE] = ellip(Ne,Rp,Rs,We,'low');

%% 6. DISPLAY INPUT PARAMETERS

fprintf('\n====================================================\n');

fprintf('       IIR FILTER COMPARATIVE ANALYSIS\n');

fprintf('====================================================\n');

fprintf('\nINPUT PARAMETERS\n');

fprintf('Sampling Frequency       = %.2f Hz\n',Fs);

fprintf('Passband Edge Frequency  = %.2f Hz\n',Fp);

fprintf('Stopband Edge Frequency  = %.2f Hz\n',Fst);

fprintf('Passband Ripple          = %.2f dB\n',Rp);

fprintf('Stopband Attenuation     = %.2f dB\n',Rs);

%% 7. CHEBYSHEV FORMULA RESULTS

fprintf('\nCHEBYSHEV TYPE-I FORMULA\n');

fprintf('Ripple factor epsilon    = %.4f\n',epsilon);

fprintf('Calculated order         = %.4f\n',N_formula);

fprintf('Theoretical minimum N    = %d\n',N_theory);

%% 8. FILTER ORDER COMPARISON

fprintf('\n====================================================\n');

fprintf('              FILTER ORDER COMPARISON\n');

fprintf('====================================================\n');

fprintf('Butterworth Order        = %d\n',Nb);

fprintf('Chebyshev Type-I Order   = %d\n',Nc);

fprintf('Elliptic Order           = %d\n',Ne);

%% 9. COEFFICIENTS

fprintf('\n====================================================\n');

fprintf('           BUTTERWORTH COEFFICIENTS\n');

fprintf('====================================================\n');

fprintf('Numerator b =\n'); disp(bB);

fprintf('Denominator a =\n'); disp(aB);

fprintf('\n====================================================\n');

fprintf('           CHEBYSHEV TYPE-I COEFFICIENTS\n');

fprintf('====================================================\n');

fprintf('Numerator b =\n'); disp(bC);

fprintf('Denominator a =\n'); disp(aC);

fprintf('\n====================================================\n');

fprintf('             ELLIPTIC COEFFICIENTS\n');

fprintf('====================================================\n');

fprintf('Numerator b =\n'); disp(bE);

fprintf('Denominator a =\n'); disp(aE);

%% 10. FREQUENCY RESPONSES

N = 4096;

[HB,f] = freqz(bB,aB,N,Fs);

[HC,~] = freqz(bC,aC,N,Fs);

[HE,~] = freqz(bE,aE,N,Fs);

MB = 20*log10(abs(HB)+eps);

MC = 20*log10(abs(HC)+eps);

ME = 20*log10(abs(HE)+eps);

%% 11. PASSBAND RIPPLE

P = f <= Fp;

Ripple_B = max(MB(P))-min(MB(P));

Ripple_C = max(MC(P))-min(MC(P));

Ripple_E = max(ME(P))-min(ME(P));

%% 12. STOPBAND ATTENUATION

S = f >= Fst;

Stop_B = -max(MB(S));

Stop_C = -max(MC(S));

Stop_E = -max(ME(S));

%% 13. PERCENTAGE DEVIATION

Dev_C_Ripple = abs(Ripple_C-Rp)/Rp*100;

Dev_B_Stop = abs(Stop_B-Rs)/Rs*100;

Dev_C_Stop = abs(Stop_C-Rs)/Rs*100;

Dev_E_Stop = abs(Stop_E-Rs)/Rs*100;

%% 14. TRANSITION CHARACTERISTICS

TransitionBW = Fst-Fp;

%% 15. DISPLAY PERFORMANCE

fprintf('\n====================================================\n');

fprintf('             FILTER PERFORMANCE\n');

fprintf('====================================================\n');

fprintf('\nPASSBAND RIPPLE\n');

fprintf('Butterworth      = %.4f dB\n',Ripple_B);

fprintf('Chebyshev Type-I = %.4f dB\n',Ripple_C);

fprintf('Elliptic         = %.4f dB\n',Ripple_E);

fprintf('\nSTOPBAND ATTENUATION\n');

fprintf('Butterworth      = %.4f dB\n',Stop_B);

fprintf('Chebyshev Type-I = %.4f dB\n',Stop_C);

fprintf('Elliptic         = %.4f dB\n',Stop_E);

fprintf('\nCHEBYSHEV RIPPLE DEVIATION\n');

fprintf('Specified Ripple = %.2f dB\n',Rp);

fprintf('Measured Ripple  = %.4f dB\n',Ripple_C);

fprintf('Percentage Deviation = %.4f %%\n',Dev_C_Ripple);

fprintf('\nTRANSITION CHARACTERISTICS\n');

fprintf('Passband Edge     = %.2f Hz\n',Fp);

fprintf('Stopband Edge     = %.2f Hz\n',Fst);

fprintf('Transition Band   = %.2f Hz\n',TransitionBW);

%% 16. MAGNITUDE RESPONSE

figure;

plot(f,MB,'LineWidth',1.5); hold on;

plot(f,MC,'LineWidth',1.5);

plot(f,ME,'LineWidth',1.5);

grid on;

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)');

title('Comparison of IIR Filter Magnitude Responses');

legend('Butterworth','Chebyshev Type-I','Elliptic');

xlim([0 250]); ylim([-80 5]);

xline(Fp,'--','Fp = 100 Hz');

xline(Fst,'--','Fst = 150 Hz');

yline(-Rp,'--','Rp = -1 dB');

yline(-Rs,'--','Rs = -40 dB');

hold off;

%% 17. PHASE RESPONSE

Phase_B = unwrap(angle(HB))*180/pi;

Phase_C = unwrap(angle(HC))*180/pi;

Phase_E = unwrap(angle(HE))*180/pi;

figure;

plot(f,Phase_B,'LineWidth',1.2); hold on;

plot(f,Phase_C,'LineWidth',1.2);

plot(f,Phase_E,'LineWidth',1.2);

grid on;

xlabel('Frequency (Hz)'); ylabel('Phase (degrees)');

title('Comparison of IIR Filter Phase Responses');

legend('Butterworth','Chebyshev Type-I','Elliptic');

xlim([0 250]);

hold off;

%% 18. FILTER ORDER GRAPH

figure;

bar([Nb Nc Ne]);

set(gca,'XTick',1:3);

set(gca,'XTickLabel',{'Butterworth','Chebyshev-I','Elliptic'});

ylabel('Filter Order'); xlabel('Filter Type');

title('Comparison of IIR Filter Orders');

grid on;

OUTPUT:

RESULT:

====================================================

IIR FILTER COMPARATIVE ANALYSIS

====================================================

INPUT PARAMETERS

Sampling Frequency       = 1000.00 Hz

Passband Edge Frequency  = 100.00 Hz

Stopband Edge Frequency  = 150.00 Hz

Passband Ripple          = 1.00 dB

Stopband Attenuation     = 40.00 dB

CHEBYSHEV TYPE-I FORMULA

Ripple factor epsilon    = 0.5088

Calculated order         = 6.2071

Theoretical minimum N    = 7

====================================================

FILTER ORDER COMPARISON

====================================================

Butterworth Order        = 12

Chebyshev Type-I Order   = 6

Elliptic Order           = 4

====================================================

BUTTERWORTH COEFFICIENTS

====================================================

Numerator b =

1.0e-03 *

0.0002    0.0027    0.0146    0.0488    0.1098    0.1757    0.2050    0.1757    0.1098    0.0488    0.0146    0.0027    0.0002

Denominator a =

1.0000   -6.8844   22.4341  -45.5100   63.7725  -64.8501   48.9643  -27.6092   11.5218   -3.4662    0.7128   -0.0899    0.0053

====================================================

CHEBYSHEV TYPE-I COEFFICIENTS

====================================================

Numerator b =

1.0e-03 *

0.0464    0.2782    0.6956    0.9274    0.6956    0.2782    0.0464

Denominator a =

1.0000   -4.8694   10.3807  -12.3367    8.5969   -3.3268    0.5586

====================================================

ELLIPTIC COEFFICIENTS

====================================================

Numerator b =

0.0197   -0.0171    0.0333   -0.0171    0.0197

Denominator a =

1.0000   -3.0330    3.8118   -2.2911    0.5554

====================================================

FILTER PERFORMANCE

====================================================

PASSBAND RIPPLE

Butterworth      = 0.8033 dB

Chebyshev Type-I = 1.0000 dB

Elliptic         = 1.0000 dB

STOPBAND ATTENUATION

Butterworth      = 40.0198 dB

Chebyshev Type-I = 41.3365 dB

Elliptic         = 40.0000 dB

CHEBYSHEV RIPPLE DEVIATION

Specified Ripple = 1.00 dB

Measured Ripple  = 1.0000 dB

Percentage Deviation = 0.0000 %

TRANSITION CHARACTERISTICS

Passband Edge     = 100.00 Hz

Stopband Edge     = 150.00 Hz

Transition Band   = 50.00 Hz

>>