SIMULATION EXPERIMENT – 9

9. Uniform Quantization and Finite Word-Length Effects in Digital Signal Processing Using MATLAB

Objectives of this Experiment:

1. To implement uniform quantization of a discrete-time signal using different word lengths.

2. To analyze quantization error produced by rounding and truncation.

3. To calculate SQNR and study the effect of word length on quantization performance.

4. To compare input/output quantization results and interpret finite word-length effects.

PROGRAM CODE:

clc; clear; close all;

%% FIR FILTER PARAMETERS

b = [0.0625 0.2500 0.3750 0.2500 0.0625];

Fs = 1000;

N = 128;

x = [1 zeros(1,N-1)];

n = 0:N-1;

%% 1. DIRECT REALIZATION

yd = filter(b,1,x);

%% 2. CASCADE REALIZATION

z = roots(b);

g = b(1);

yc = g*x;

for k = 1:length(z)

yc = filter([1 -z(k)],1,yc);

end

yc = real(yc);

%% 3. SYMMETRY AND LINEAR PHASE

symErr = max(abs(b-fliplr(b)));

D = (length(b)-1)/2;

if symErr < 1e-12

symmetry = 'Symmetric';

else

symmetry = 'Not Symmetric';

end

%% 4. FREQUENCY, PHASE AND GROUP DELAY

[H,w] = freqz(b,1,1024);

phase = unwrap(angle(H));

gd = grpdelay(b,1,1024);

%% 5. NUMERICAL COMPARISON

err = max(abs(yd-yc));

dev = err/max(abs(yd))*100;

%% =========================================================

% DISPLAY RESULTS

% =========================================================

fprintf('\n============================================\n');

fprintf('       FIR FILTER REALIZATION RESULTS\n');

fprintf('============================================\n');

fprintf('\nFIR Coefficients:\n');

disp(b);

fprintf('Sampling Frequency = %d Hz\n',Fs);

fprintf('Number of Samples  = %d\n',N);

fprintf('Coefficient Symmetry = %s\n',symmetry);

fprintf('Symmetry Error       = %.3e\n',symErr);

fprintf('Group Delay          = %.2f samples\n',D);

fprintf('\nMaximum Error        = %.3e\n',err);

fprintf('Percentage Deviation = %.3e %%\n',dev);

%% OUTPUT PARAMETER TABLE

Parameter = {

'Direct realization output'

'Cascade realization output'

'Impulse response'

'Phase response'

'Comparison of realizations'

};

Observation = {

'Direct FIR filtering'

'Cascade of first-order sections'

'Equal to FIR coefficient sequence'

'Linear phase'

'Equivalent transfer-function behavior'

};

Value = {

sprintf('[%.4f %.4f %.4f %.4f %.4f]',yd(1:5))

sprintf('[%.4f %.4f %.4f %.4f %.4f]',yc(1:5))

sprintf('[%.4f %.4f %.4f %.4f %.4f]',yd(1:5))

sprintf('Group delay = %.2f samples',D)

sprintf('Max error = %.3e; Deviation = %.3e %%',err,dev)

};

Unit_Note = {

'Amplitude'

'Amplitude'

'h[n]'

'Samples'

'Numerical precision'

};

Output_Table = table(Parameter,Observation,Value,Unit_Note);

fprintf('\n============================================\n');

fprintf('OUTPUT PARAMETER TABLE\n');

fprintf('============================================\n');

disp(Output_Table);

%% PLOTTING

figure('Name', 'Direct vs Cascade Impulse Response');

stem(n,yd,'filled'); hold on;

stem(n,yc,'--');

grid on; xlabel('Sample n'); ylabel('Amplitude');

title('Direct and Cascade FIR Impulse Responses');

legend('Direct Realization','Cascade Realization');

figure('Name', 'FIR Frequency and Phase Response');

subplot(2,1,1);

plot(w/pi,abs(H),'LineWidth',1.5); grid on;

xlabel('Normalized Frequency (\times\pi rad/sample)'); ylabel('|H(e^{j\omega})|');

title('FIR Magnitude Response');

subplot(2,1,2);

plot(w/pi,phase,'LineWidth',1.5); grid on;

xlabel('Normalized Frequency (\times\pi rad/sample)'); ylabel('Phase (radians)');

title('Unwrapped Phase Response');

figure('Name', 'Group Delay');

plot(w/pi,gd,'LineWidth',1.5); grid on;

xlabel('Normalized Frequency (\times\pi rad/sample)'); ylabel('Group Delay (samples)');

title('Group Delay');

figure('Name', 'Numerical Error');

plot(n,abs(yd-yc),'LineWidth',1.5); grid on;

xlabel('Sample n'); ylabel('Absolute Error');

title('Direct vs Cascade Numerical Error');
OUTPUT:

RESULT:

========== QUANTIZATION RESULTS ==========

Amplitude = 1.0 | Frequency = 50 Hz | Fs = 1000 Hz

Samples = 1000 | Main word length = 8 bits

8-bit Step Size = 0.0078125

Rounding SQNR = 48.9339 dB

Truncation SQNR = 45.2931 dB

Output SQNR = 48.9339 dB

Max Rounding Error = 0.003483

Max Truncation Error = 0.005744

Bits     StepSize     SQNR_Round_dB    SQNR_Trunc_dB    MaxError_Round    MaxError_Trunc

____    __________    _____________    _____________    ______________    ______________

4          0.125       23.667           20.871            0.059017          0.087785

8      0.0078125       48.934           45.293            0.003483          0.005744

12     0.00048828       78.811           65.891          0.00011536        0.00042324

16     3.0518e-05       99.697           91.599           1.382e-05        2.6516e-05

Parameter                                Observation                            Unit

__________________________    __________________________________________________    _____________

{'Quantized signal'      }    {'8-bit rounding, Delta = 0.0078125'             }    {'Amplitude'}

{'Quantization error'    }    {'Max rounding error = 0.003483'                 }    {'Amplitude'}

{'SQNR'                  }    {'Rounding = 48.9339 dB; Truncation = 45.2931 dB'}    {'dB'       }

{'Word-length comparison'}    {'SQNR increases as word length increases'       }    {'Bits / dB'}

>>