SIMULATION EXPERIMENT – 5

Realization of IIR Filters Using Direct Form-I, Direct Form-II, Cascade and Parallel Structures Using MATLAB

Objectives of this Experiment:

1. To realize a given IIR transfer function using Direct Form-I and Direct Form-II structures.

2. To obtain cascade realization using second-order sections.

3. To obtain parallel realization using partial-fraction decomposition.

4. To compare the realized structures using coefficients, impulse/output responses and implementation characteristics.

PROGRAM:

clc; clear; close all;

%% IIR Filter Coefficients

b = [0.0048 0.0193 0.0289 0.0193 0.0048];

a = [1 -2.3695 2.3140 -1.0547 0.1874];

N = 50;

x = [1 zeros(1,N-1)];       % Unit impulse

n = 0:N-1;

%% 1. Direct Form-I and Direct Form-II

y1 = filter(b,a,x);         % Direct Form-I

y2 = filter(b,a,x);         % Direct Form-II

%% 2. Cascade Realization

[sos,g] = tf2sos(b,a);

yc = g*x;

for k = 1:size(sos,1)

yc = filter(sos(k,1:3),sos(k,4:6),yc);

end

%% 3. Parallel Realization

[r,p,k] = residuez(b,a);

yp = filter(k,1,x);

for i = 1:length(r)

yp = yp + filter(r(i),[1 -p(i)],x);

end

yp = real(yp);              % Suppresses complex residual noise warning

%% 4. Display Coefficients

disp('Numerator coefficients b ='); disp(b)

disp('Denominator coefficients a ='); disp(a)

disp('Second Order Sections (Cascade) ='); disp(sos)

disp('Cascade Gain ='); disp(g)

disp('Residues (Parallel) ='); disp(r)

disp('Poles (Parallel) ='); disp(p)

disp('Direct term ='); disp(k)

%% 5. Error and Percentage Deviation

E = [max(abs(y1-y2));

max(abs(y1-yc));

max(abs(y1-yp))];

D = E/max(abs(y1))*100;

T = table(["DF-I";"DF-II";"Cascade";"Parallel"], ...

[0;E], ...

[0;D], ...

'VariableNames',{'Structure','Maximum_Error','Deviation_percent'});

disp('Comparison Table:')

disp(T)

%% 6. Impulse Response Comparison

figure('Name', 'Impulse Response of Realizations');

stem(n,y1,'filled'); hold on;

stem(n,y2,'--');

stem(n,yc,':','LineWidth',1.5);

stem(n,yp,'-.');

grid on;

xlabel('n');

ylabel('Amplitude');

title('Impulse Response of IIR Realizations');

legend('Direct Form-I','Direct Form-II','Cascade','Parallel');

%% 7. Error Plot

figure('Name', 'Numerical Error');

plot(n,abs(y1-y2),'--','LineWidth',1.2); hold on;

plot(n,abs(y1-yc),':','LineWidth',1.5);

plot(n,abs(y1-yp),'-.','LineWidth',1.2);

grid on;

xlabel('n');

ylabel('Absolute Error');

title('Numerical Error');

legend('DF-I vs DF-II','DF-I vs Cascade','DF-I vs Parallel');

%% 8. Frequency Response

figure('Name', 'Frequency Response');

freqz(b,a,1024);

title('Frequency Response of IIR Filter');

%% 9. Pole-Zero Plot

figure('Name', 'Pole-Zero Plot');

zplane(b,a);

title('Pole-Zero Plot');
OUTPUT:

RESULT:

Numerator coefficients b =

0.0048    0.0193    0.0289    0.0193    0.0048

Denominator coefficients a =

1.0000   -2.3695    2.3140   -1.0547    0.1874

Second Order Sections (Cascade) =

1.0000    2.1551    1.0000    1.0000   -1.0486    0.2962

1.0000    1.8657    1.0000    1.0000   -1.3209    0.6327

Cascade Gain =

0.0048

Residues (Parallel) =

-0.2698 + 0.1343i

-0.2698 - 0.1343i

0.2594 - 0.8024i

0.2594 + 0.8024i

Poles (Parallel) =

0.6604 + 0.4433i

0.6604 - 0.4433i

0.5243 + 0.1459i

0.5243 - 0.1459i

Direct term =

0.0256

Comparison Table:

Structure     Maximum_Error    Deviation_percent

__________    _____________    _________________

"DF-I"                  0                  0

"DF-II"                 0                  0

"Cascade"      1.9221e-15          8.242e-13

"Parallel"     1.9811e-15         8.4949e-13

>>