SIMULATION EXPERIMENT – 1

1.Design, Realization and Performance Analysis of Butterworth IIR Low-Pass and High-Pass Digital Filters Using MATLAB

Objectives of this Experiment:

1. To design Butterworth IIR low-pass and high-pass digital filters for specified frequency requirements using MATLAB.

2. To obtain and verify the filter coefficients and determine the required filter order.

3. To plot and analyze the magnitude and phase responses of the designed filters.

4. To identify the cutoff frequency and examine the passband and stopband characteristics.

Program Code:

clear;

clc;

close all;

% Design Parameters

Fs = 8000;          % Sampling frequency in Hz

Ap = 1;             % Passband ripple in dB

As = 40;            % Stopband attenuation in dB

% 1. Low-Pass Filter Design

fp_lp = 1000;       % LPF Passband frequency (Hz)

fs_lp = 1500;       % LPF Stopband frequency (Hz)

wp_lp = fp_lp / (Fs/2);

ws_lp = fs_lp / (Fs/2);

[N_lp, Wn_lp] = buttord(wp_lp, ws_lp, Ap, As);

[b_lp, a_lp] = butter(N_lp, Wn_lp, 'low');

% 2. High-Pass Filter Design

fp_hp = 2500;       % HPF Passband frequency (Hz)

fs_hp = 1800;       % HPF Stopband frequency (Hz)

wp_hp = fp_hp / (Fs/2);

ws_hp = fs_hp / (Fs/2);

[N_hp, Wn_hp] = buttord(wp_hp, ws_hp, Ap, As);

[b_hp, a_hp] = butter(N_hp, Wn_hp, 'high');

% ==========================================

% DISPLAY NUMERICAL OUTPUTS IN COMMAND WINDOW

% ==========================================

fprintf('=== EXPERIMENT 1: BUTTERWORTH IIR FILTERS ===\n\n');

fprintf('--- LOW-PASS FILTER ---\n');

fprintf('Calculated Filter Order (N_lp): %d\n', N_lp);

fprintf('3-dB Cutoff Frequency (Wn_lp): %.4f Hz\n', Wn_lp * (Fs/2));

fprintf('Numerator Coefficients (b_lp):\n');

disp(b_lp);

fprintf('Denominator Coefficients (a_lp):\n');

disp(a_lp);

fprintf('--- HIGH-PASS FILTER ---\n');

fprintf('Calculated Filter Order (N_hp): %d\n', N_hp);

fprintf('3-dB Cutoff Frequency (Wn_hp): %.4f Hz\n', Wn_hp * (Fs/2));

fprintf('Numerator Coefficients (b_hp):\n');

disp(b_hp);

fprintf('Denominator Coefficients (a_hp):\n');

disp(a_hp);

% Frequency Response Calculation

[H_lp, f] = freqz(b_lp, a_lp, 1024, Fs);

[H_hp, ~] = freqz(b_hp, a_hp, 1024, Fs);

% Plotting

figure('Name', 'Butterworth IIR LPF and HPF');

subplot(2,2,1);

plot(f, 20*log10(abs(H_lp)), 'b', 'LineWidth', 1.5); grid on;

title(['Butterworth LPF Magnitude (Order N = ', num2str(N_lp), ')']);

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)'); ylim([-80 5]);

subplot(2,2,2);

plot(f, angle(H_lp)*180/pi, 'r', 'LineWidth', 1.5); grid on;

title('Butterworth LPF Phase Response');

xlabel('Frequency (Hz)'); ylabel('Phase (degrees)');

subplot(2,2,3);

plot(f, 20*log10(abs(H_hp)), 'b', 'LineWidth', 1.5); grid on;

title(['Butterworth HPF Magnitude (Order N = ', num2str(N_hp), ')']);

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)'); ylim([-80 5]);

subplot(2,2,4);

plot(f, angle(H_hp)*180/pi, 'r', 'LineWidth', 1.5); grid on;

title('Butterworth HPF Phase Response');

xlabel('Frequency (Hz)'); ylabel('Phase (degrees)');

Output:

RESULT:

=== EXPERIMENT 1: BUTTERWORTH IIR FILTERS ===

--- LOW-PASS FILTER ---

Calculated Filter Order (N_lp): 12

3-dB Cutoff Frequency (Wn_lp): 1087.8354 Hz

Numerator Coefficients (b_lp):

0.0000    0.0000    0.0002    0.0006    0.0013    0.0021    0.0024    0.0021    0.0013    0.0006    0.0002    0.0000    0.0000

Denominator Coefficients (a_lp):

1.0000   -5.4622   14.6771  -25.1616   30.3497  -26.9537   17.9862   -9.0534    3.4018   -0.9283    0.1743   -0.0202    0.0011

--- HIGH-PASS FILTER ---

Calculated Filter Order (N_hp): 10

3-dB Cutoff Frequency (Wn_hp): 2379.7556 Hz

Numerator Coefficients (b_hp):

0.0006   -0.0055    0.0248   -0.0662    0.1159   -0.1390    0.1159   -0.0662    0.0248   -0.0055    0.0006

Denominator Coefficients (a_hp):

1.0000    1.8915    2.8535    2.6014    1.8704    0.9534    0.3726    0.1030    0.0199    0.0023    0.0001

>>