10. Investigation of Coefficient and Product Quantization Noise and Limit-Cycle Oscillations Using MATLAB
Objectives of this Experiment:

1. To investigate coefficient quantization and product quantization in a digital filter.

2. To measure the effect of finite word length on the filter response and output signal.

3. To demonstrate zero-input and overflow limit-cycle oscillations.

4. To evaluate the resulting quantization noise and performance degradation.

PROGRAM CODE:

clc; clear; close all;

%% INPUTS

b = [0.0048 0.0193 0.0289 0.0193 0.0048];

a = [1 -2.3695 2.3140 -1.0547 0.1874];

Bc = 8;                 % Coefficient word length

Bp = 16;                % Product word length

N  = 200;

x  = [1 zeros(1,N-1)];  % Unit impulse

%% 1. COEFFICIENT QUANTIZATION

S = 2^(Bc-1)-1;

bq = round(b*S)/S;

aq = round(a*S)/S;

CoeffError = [b-bq, a-aq];

%% 2. IDEAL AND COEFFICIENT-QUANTIZED RESPONSE

y_ideal = filter(b,a,x);

y_coeff = filter(bq,aq,x);

e_coeff = y_ideal - y_coeff;

MaxCoeffError = max(abs(e_coeff));

SQNR_coeff = 10*log10(sum(y_ideal.^2)/sum(e_coeff.^2));

%% 3. PRODUCT QUANTIZATION

Sp = 2^(Bp-1)-1;

y_prod = zeros(1,N);

v = zeros(1,4);

for n = 1:N

p = bq*x(n);

fb = -aq(2:5).*v;

p = round([p fb]*Sp)/Sp;    % Product quantization

y_prod(n) = sum(p);

v = [y_prod(n) v(1:3)];

end

e_prod = y_coeff - y_prod;

MaxProdError = max(abs(e_prod));

SQNR_prod = 10*log10(sum(y_coeff.^2)/sum(e_prod.^2));

%% 4. ZERO-INPUT LIMIT-CYCLE TEST

x0 = zeros(1,N);

state = [0.01 0.01 0.01 0.01];

y_zero = zeros(1,N);

for n = 1:N

acc = -aq(2:5)*state.';

y_zero(n) = round(acc*Sp)/Sp;

state = [y_zero(n) state(1:3)];

end

LimitAmplitude = max(abs(y_zero));

%% 5. OVERFLOW / SATURATION TEST

xo = 1.2*ones(1,N);

y_over = filter(bq,aq,xo);

y_sat = min(max(y_over,-1),1);   % Normalized saturation

OverflowCount = sum(abs(y_over)>1);

MaxOverflow = max(abs(y_over));

%% 6. OUTPUT SUMMARY DISPLAY

fprintf('\n--- QUANTIZATION RESULTS ---\n');

fprintf('Maximum coefficient/output error = %.6e\n', MaxCoeffError);

fprintf('Coefficient SQNR                 = %.2f dB\n', SQNR_coeff);

fprintf('Maximum product quantization error = %.6e\n', MaxProdError);

fprintf('Product SQNR                     = %.2f dB\n', SQNR_prod);

fprintf('Zero-input maximum amplitude     = %.6e\n', LimitAmplitude);

fprintf('Overflow samples                 = %d\n', OverflowCount);

T = table( ...

["Coefficient Quantization"; "Product Quantization"; "Zero-Input Test"; "Overflow Test"], ...

[MaxCoeffError; MaxProdError; LimitAmplitude; MaxOverflow], ...

[SQNR_coeff; SQNR_prod; NaN; NaN], ...

'VariableNames', {'Test', 'Maximum_Error_or_Amplitude', 'SQNR_dB'});

disp(T);

%% 7. COEFFICIENTS DISPLAY

disp('Original b ='); disp(b);

disp('Quantized b ='); disp(bq);

disp('Original a ='); disp(a);

disp('Quantized a ='); disp(aq);

%% 8. PLOTS

figure('Name', 'Ideal vs Quantized Impulse Response');

stem(0:N-1, y_ideal, 'filled'); hold on;

stem(0:N-1, y_coeff);

grid on; xlabel('n'); ylabel('Amplitude');

title('Ideal vs Coefficient-Quantized Response');

legend('Ideal','Quantized');

figure('Name', 'Quantization Noises');

subplot(2,1,1);

plot(0:N-1, e_coeff, 'LineWidth', 1);

grid on; xlabel('n'); ylabel('Error');

title('Coefficient Quantization Noise');

subplot(2,1,2);

stem(0:N-1, e_prod, 'filled');

grid on; xlabel('n'); ylabel('Error');

title('Product Quantization Noise');

figure('Name', 'Zero-Input Limit Cycle');

plot(0:N-1, y_zero, 'LineWidth', 1);

grid on; xlabel('n'); ylabel('Amplitude');

title('Zero-Input Limit-Cycle Test');

figure('Name', 'Saturation Test');

plot(0:N-1, y_over); hold on;

plot(0:N-1, y_sat, '--');

grid on; xlabel('n'); ylabel('Amplitude');

title('Overflow / Saturation Test');

legend('Without Saturation','With Saturation');

figure('Name', 'Quantized Pole-Zero Plot');

zplane(bq, aq); grid on;

title('Quantized Filter Pole-Zero Plot');
OUTPUT:

RESULT:

--- QUANTIZATION RESULTS ---

Maximum coefficient/output error = 4.405764e-03

Coefficient SQNR                 = 32.23 dB

Maximum product quantization error = 1.651951e-01

Product SQNR                     = 1.57 dB

Zero-input maximum amplitude     = 9.216590e-03

Overflow samples                 = 194

Test               Maximum_Error_or_Amplitude    SQNR_dB

__________________________    __________________________    _______

"Coefficient Quantization"            0.0044058             32.233

"Product Quantization"                   0.1652             1.5673

"Zero-Input Test"                     0.0092166                NaN

"Overflow Test"                          1.3599                NaN

Original b =

0.0048    0.0193    0.0289    0.0193    0.0048

Quantized b =

0.0079    0.0157    0.0315    0.0157    0.0079

Original a =

1.0000   -2.3695    2.3140   -1.0547    0.1874

Quantized a =

1.0000   -2.3701    2.3150   -1.0551    0.1890

>>